Experiment 1.3

- Hub

A hub id basically a multi-port repeater. A hub connects multiple wires coming form different branches for example, the connector in star topology which connects different stations. Hub cannot filter data, so data packets are sent to all connected device. In other words, the collision domain of all hosts connected through Hub remain one. Also they do not have the intelligence to find out the best path for data packets which leads to inefficiencies and wastage.

- Switch

A switch is a multi-port bridge with a buffer and a design that can boost its efficiency (a large number of ports imply less traffic) and performance. A switch is a data link layer device. The switch can perform error checking before forwarding data, which makes it very efficient as it does not forward packets that have errors and forward good packets selectively to the correct port only. In other words, the switch divides the collision domain of hosts, but broadcast domain remains the same.

- Router

A router is a device that connects two or more packets-switched networks or sub-networks. It serves two primary functions: managing traffic between these network by forwarding data packets to their intended IP addresses, and allowing multiple devices to use the same internet connection.

Experiment 2.1

- Star topology

It is the type of topology in which all the system are connected to a single hub and the information is subjected to reach each and every component of the system. These are like client/ server network.

- Bus

It is the simplest topology in which all the devices are connected to a single backbone cable and it controls the data flow in the network system. Here all the devices are connected in bus topology and follow all the protocols of a network system to be connected in the bus topology.

Experiment 1.2

- Ping

- n the context of computer networks, "ping" refers to a network utility used to test the reachability of a host or network device and measure the time it takes for a packet to travel from the source to the destination and back. The term "ping" originates from sonar technology, where a sound pulse is emitted and the echo is used to determine the distance to an object. Similarly, the network ping sends a small packet of data to a target IP address or hostname and waits for the response.

- The ping command sends Internet Control Message Protocol (ICMP) Echo Request messages to the destination device, which then responds with an ICMP Echo Reply message if it is reachable. By measuring the time it takes for the request and reply to travel back and forth, the ping utility provides an estimate of the round-trip time (RTT) between the source and destination. This information is useful for diagnosing network connectivity issues, assessing network performance, and troubleshooting network problems.

- Ipconfig

The "ipconfig" command is a Windows command-line utility used to display the IP configuration settings of a computer's network interfaces. It provides information such as IP addresses, subnet masks, default gateways, and DNS server addresses. Here's how to use the "ipconfig" command:

- Tracert

The "tracert" command (short for "traceroute") is a network diagnostic tool available in Windows command prompt that allows you to trace the route taken by packets from your computer to a destination IP address or hostname. It shows the intermediate network devices (routers) that the packets traverse and the time it takes for the packets to reach each device. Here's how to use the "tracert" command:

- Open a command prompt window on your Windows computer. You can do this by pressing the Windows key, typing "Command Prompt," and selecting the "Command Prompt" application from the search results.

- In the command prompt window, type the following command:

phpCopy code

tracert <destination>

Replace <destination> with the IP address or hostname of the target device or website you want to trace. For example, you can use a domain name like tracert www.example.com or an IP address like tracert 192.168.0.1.

- Press Enter to execute the command.

- The "tracert" command will start sending ICMP Echo Request messages with increasing time-to-live (TTL) values. Each packet will expire at a different router along the path to the destination, and the router will send back an ICMP Time Exceeded message to indicate the hop where the packet expired.

- The command prompt will display a list of the intermediate routers (along with their IP addresses) and the round-trip time (in milliseconds) it took for the packets to reach each router. It will also show the final destination IP address or hostname at the end of the trace route.

- The output will typically include information such as the number of hops (routers) involved, their IP addresses, and the response time for each hop.

- By default, the "tracert" command sends three ICMP Echo Requests to each router along the path. You can modify the number of packets or other options depending on your requirements. To learn more about the available options, you can type tracert /? in the command prompt.

Note that the "tracert" command uses ICMP Echo Requests, which may be blocked by firewalls or routers. In such cases, you may see asterisks (*) or "Request Timed Out" messages for certain hops. Additionally, the response times displayed are approximate and may vary due to network congestion or other factors.

- Nslookup

The "nslookup" command is a network tool available in command prompt or terminal that allows you to query DNS (Domain Name System) servers to retrieve information about domain names, IP addresses, and DNS records. It can be used to troubleshoot DNS-related issues, resolve domain names to IP addresses, and obtain DNS information for a specific domain. Here's how to use the "nslookup" command:

- Open a command prompt or terminal window on your computer. The method for doing this varies depending on your operating system:

  - On Windows: Press the Windows key, type "Command Prompt," and click on the "Command Prompt" or "Windows PowerShell" application.

  - On macOS: Open the "Applications" folder, go to "Utilities," and launch the "Terminal" application.

  - On Linux: Open the "Terminal" application from your distribution's menu or use the keyboard shortcut Ctrl+Alt+T.

- In the command prompt or terminal window, type the following command:

phpCopy code

nslookup <domain>

Replace <domain> with the domain name or IP address you want to look up. For example, you can perform a DNS lookup for a domain like nslookup www.example.com or an IP address like nslookup 8.8.8.8.

- Press Enter to execute the command.

- The "nslookup" command will send a DNS query to the configured DNS server and display the results. The output typically includes the IP address(es) associated with the domain name, the authoritative DNS server(s) for the domain, and additional DNS records such as MX (Mail Exchanger) records or TXT (Text) records.

- By default, the "nslookup" command queries the default DNS server configured on your computer. If you want to query a specific DNS server, you can specify its IP address after the domain name. For example:

Copy code

nslookup www.example.com 8.8.8.8

This command will perform a DNS lookup for "www.example.com" using the DNS server at IP address 8.8.8.8 (which is one of Google's public DNS servers).

- You can also use other options with the "nslookup" command to perform specific types of queries or retrieve more detailed information. To learn about these options, you can type nslookup -h or nslookup /? in the command prompt or terminal.

The "nslookup" command is available on Windows, macOS, and Linux systems, although the specific output and available options may vary slightly between platforms.

  - Netstat

The "netstat" command is a network utility available in command prompt or terminal that displays active network connections, listening ports, and various network statistics on a computer. It provides information about established connections, listening ports, routing tables, and other network-related details. Here's how to use the "netstat" command:

- Open a command prompt or terminal window on your computer. The method for doing this varies depending on your operating system:

  - On Windows: Press the Windows key, type "Command Prompt," and click on the "Command Prompt" or "Windows PowerShell" application.

  - On macOS: Open the "Applications" folder, go to "Utilities," and launch the "Terminal" application.

  - On Linux: Open the "Terminal" application from your distribution's menu or use the keyboard shortcut Ctrl+Alt+T.

- In the command prompt or terminal window, type the following command:

Copy code

netstat

By default, this command will display a list of active network connections (both incoming and outgoing) along with their local and remote IP addresses and port numbers.

- Press Enter to execute the command.

- The "netstat" command will provide information about active connections, including the protocol used (TCP or UDP), the local IP address and port, the remote IP address and port, and the connection status (e.g., ESTABLISHED, LISTENING, TIME_WAIT, etc.).

- You can use various options with the "netstat" command to display specific types of information or filter the output based on your requirements. Some common options include:

  - -a: Displays all connections and listening ports, including those in the LISTENING state.

  - -n: Displays numerical IP addresses and port numbers instead of resolving them to hostnames and service names.

  - -b (Windows): Displays the executable responsible for establishing each connection or listening port.

  - -p <protocol>: Displays connections for a specific protocol, such as TCP or UDP.

To use these options, append them to the "netstat" command. For example, to display all active connections and listening ports with numerical addresses and ports, you would use the following command:

cssCopy code

netstat -a -n

- The specific output and available options may vary slightly between different operating systems. To learn about the available options for your system, you can type netstat --help or netstat -h in the command prompt or terminal.

The "netstat" command provides valuable information for network troubleshooting, monitoring network activity, and identifying potential network-related issues on your computer.

  - Arp

The "arp" command is a network utility available in command prompt or terminal that allows you to view and manipulate the ARP (Address Resolution Protocol) cache in your operating system. ARP is used to map IP addresses to MAC (Media Access Control) addresses on a local network. The "arp" command provides information about the ARP cache and can be used to manually add or remove ARP entries. Here's how to use the "arp" command:

- Open a command prompt or terminal window on your computer. The method for doing this varies depending on your operating system:

  - On Windows: Press the Windows key, type "Command Prompt," and click on the "Command Prompt" or "Windows PowerShell" application.

  - On macOS: Open the "Applications" folder, go to "Utilities," and launch the "Terminal" application.

  - On Linux: Open the "Terminal" application from your distribution's menu or use the keyboard shortcut Ctrl+Alt+T.

- In the command prompt or terminal window, type the following command:

Copy code

arp

By default, this command will display the current ARP cache entries, including the IP addresses and corresponding MAC addresses.

- Press Enter to execute the command.

- The "arp" command will provide information about the ARP cache entries, including the IP addresses and their associated MAC addresses. The output typically includes columns for "Interface," "Internet Address" (IP address), "Physical Address" (MAC address), and "Type" (e.g., dynamic or static).

- You can use various options with the "arp" command to perform specific tasks or manipulate the ARP cache. Some common options include:

  - -a: Displays the complete ARP cache, including both dynamic and static entries.

  - -d <ipaddress>: Deletes a specific entry from the ARP cache. Replace <ipaddress> with the IP address you want to remove.

  - -s <ipaddress> <macaddress>: Adds a static ARP entry to the cache. Replace <ipaddress> with the IP address and <macaddress> with the MAC address you want to add.

To use these options, append them to the "arp" command. For example, to display the complete ARP cache, you would use the following command:

cssCopy code

arp -a

- The specific output and available options may vary slightly between different operating systems. To learn about the available options for your system, you can type arp --help or arp -h in the command prompt or terminal.

The "arp" command is commonly used for troubleshooting network connectivity issues, verifying IP-to-MAC address mappings, and managing the ARP cache on a local network.

  - Rarp

- The Reverse Address Resolution Protocol (RARP) is an obsolete network protocol that was used in the early days of computer networks to obtain an IP address based on a known MAC address. However, RARP has been largely replaced by more advanced protocols like DHCP (Dynamic Host Configuration Protocol). As a result, modern operating systems and network equipment no longer provide built-in support for RARP.

- Therefore, the "rarp" command is not typically available in command prompt or terminal on modern systems. You would not find a native "rarp" command to perform RARP-related tasks.

- If you need to assign IP addresses based on MAC addresses in a network environment, it is recommended to use DHCP, which is the standard protocol used for automatic IP address assignment. DHCP allows for flexible and dynamic IP address allocation, making it more suitable for modern networks.

- If you have a specific requirement or scenario related to RARP, please provide more details, and I'll be happy to assist you further.

  - Hostname

The "hostname" command is a utility available in command prompt or terminal that displays or sets the hostname of a computer or device. The hostname is a label assigned to a device on a network and is used to identify it. Here's how to use the "hostname" command:

- Open a command prompt or terminal window on your computer. The method for doing this varies depending on your operating system:

  - On Windows: Press the Windows key, type "Command Prompt," and click on the "Command Prompt" or "Windows PowerShell" application.

  - On macOS: Open the "Applications" folder, go to "Utilities," and launch the "Terminal" application.

  - On Linux: Open the "Terminal" application from your distribution's menu or use the keyboard shortcut Ctrl+Alt+T.

- In the command prompt or terminal window, type the following command:

Copy code

hostname

- Press Enter to execute the command.

- The "hostname" command will display the current hostname of your computer or device.

If you want to change the hostname, the process varies depending on the operating system you are using:

- Windows:

  - To change the hostname temporarily (until the next restart), use the following command:

phpCopy code

hostname <new_hostname>

Replace <new_hostname> with the desired hostname.

  - To change the hostname permanently, you need to access the System Properties dialog. You can do this by right-clicking on "This PC" or "My Computer," selecting "Properties," and then clicking on "Change settings" or "Advanced system settings." In the "System Properties" dialog, go to the "Computer Name" tab and click on the "Change" button. Enter the new hostname and click "OK" to save the changes. You will need administrative privileges to perform this action.

- macOS:

  - To change the hostname temporarily (until the next restart), use the following command:

arduinoCopy code

sudo scutil --set HostName <new_hostname>

Replace <new_hostname> with the desired hostname. You will need administrative privileges to perform this action.

  - To change the hostname permanently, go to "System Preferences" > "Sharing." In the "Computer Name" field, enter the new hostname. Close the preferences window to save the changes.

- Linux:

  - The process for changing the hostname can vary depending on the Linux distribution you are using. In many cases, you can edit the "/etc/hostname" file using a text editor and replace the existing hostname with the desired hostname. Save the file and restart the system for the changes to take effect. Alternatively, some distributions provide specific commands or tools to change the hostname, such as "hostnamectl" on systems using systemd.

Keep in mind that changing the hostname may require administrative privileges, and in some cases, a system restart might be necessary for the changes to take effect.

- Pathping

The "pathping" command is a network utility available in Windows command prompt that combines the functionality of "ping" and "tracert" commands. It provides information about network latency and packet loss at each hop along the network path between your computer and a destination IP address or hostname. Here's how to use the "pathping" command:

- Open a command prompt window on your Windows computer. You can do this by pressing the Windows key, typing "Command Prompt," and selecting the "Command Prompt" or "Windows PowerShell" application from the search results.

- In the command prompt window, type the following command:

phpCopy code

pathping <destination>

Replace <destination> with the IP address or hostname of the target device or website you want to analyze. For example, you can use a domain name like pathping www.example.com or an IP address like pathping 192.168.0.1.

- Press Enter to execute the command.

- The "pathping" command will start sending packets to the destination IP address or hostname and collect statistics about each hop along the network path.

- The command prompt will display detailed information, including the number of hops, round-trip times, packet loss statistics, and other network-related metrics for each hop.

- Unlike the "tracert" command, "pathping" sends multiple ICMP Echo Request packets to each hop in the path and collects statistics over a period of time (by default, 25 seconds). This provides a more comprehensive analysis of network performance, including latency and packet loss.

- The output of "pathping" includes data for each hop, as well as a summary at the end that shows the average, minimum, and maximum latency, as well as packet loss statistics.

- By default, the "pathping" command runs continuously until you manually stop it by pressing Ctrl+C. You can modify the duration of the test or other options by specifying additional parameters. To learn more about the available options, you can type pathping /? in the command prompt.

Please note that running "pathping" generates network traffic and may take some time to complete, especially if there are many hops between your computer and the destination. Additionally, firewalls or network security measures may impact the results by filtering or prioritizing certain types of traffic.